# NOTE

- Every time when you want to use IRSIM and Hspice, in the console, you have to type

setenv CAD_HOME /opt2/magic.old

alias irsim /opt2/magic.old/bin/irsim scmos1_2.prm

source cshrc-ee555

Now it is better to build a *.bat file and store the three commands above in that bat file. If you have such a bat file with the name, for example, test.bat, when you want to use Hspice or IRSIM, you only have to type the following in the console :

source test.bat

It will automatically execute those three commands. It is much simpler and easier for you. I have already built this file under the ee355 folder. You can just copy this file to your folder.

cp ../../test.bat .

- When using IRSIM, in order to see the waveform, you have to type the nodal analysis commands in the irsim console. For example,

ana ina inb outa

h vdd

l gnd

l ina inb

s 20

h ina

s 20

l ina

h inb

s 20

h ina

s 20

Now you can build a *.cmd file and store the nodal analysis commands in that

cmd file. For example, I have a cmd file named nand.cmd in my current folder.

After running

irsim nand.sim

in the irsim console, I only have to type

nand

and the IRSIM waveform analysis window will pop up automatically and show you the

waveforms I typed in the nand.cmd file.

EE355 TA

Chin